| REPORTING PARAMETERS FOR FINITE ELEMENT ANALYSIS STUDIES IN BIOMECHANICS Document: Yao et al., J. Biomech. Eng., 128: 135-141, 2006 - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 1: Model Identification |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Model name |  | X |  |
|  | Model keywords |  | X |  |
|  | Version |  | X |  |
|  | Physiological domain | X |  | Motion and deformation |
|  | Mechanical domain |  | X | Implied as quasi-static nonlinear solid mechanics based on model description |
|  | Structure of interest | X |  | Medial meniscus and relevant compartment of the knee |
|  | Demographics | X |  | Human, female, 65 years old |
|  | State of represented organism | X |  | Cadaver, in situ |
|  | Disease state | X |  | Induced dissection of anterior cruciate ligament; otherwise no significant evidence of OA, meniscal lesions, ligamentaneous abnormalities |
|  | Spatial scale | X |  | Organ/joint |
|  | Time scale |  | X | Implication of a search for static response based on utilized simulation software |
|  | Primary utility | X |  | Prediction of medial meniscal position for given tibiofemoral joint kinematics, which was obtained from MR images of an ACL deficient knee under anterior tibial loads |
|  | Secondary utility | X |  | Prediction of stress state in medial meniscus of an ACL deficient knee under anterior loading |
|  | Primary highlight | X |  | Visualize and clarify the role of the medial meniscus as a stabilizing structure in the ACL deficient knee |
|  | Secondary highlight | X |  | First model to include peripheral attachments of the meniscus to the tibia including the horn attachments of the meniscus |
|  | Primary limitation | X |  | A single specimen study without inclusion of surrounding tissue restraint (MCL, posterior musculature, joint capsule, transverse ligament); results limited to the specified experimental loading states |
|  | Secondary limitation | X |  | Inability to predict meniscal deformations in the anterior and posterior regions with accuracy less than digitizing error |

| REPORTING PARAMETERS FOR FINITE ELEMENT ANALYSIS STUDIES IN BIOMECHANICS (cont.) - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 1: Model Identification (cont.) |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Reference to publication(s) | X |  | Reporting is the publication: J. Biomech. Eng., 128: 135-141, 2006 |
|  | Pointer to publication | X |  | doi:10.1115/1.2132373 |
|  | Related models |  | X |  |
| Group 2: Model Structure |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Loading and boundary conditions for each, fill in form Reporting Parameters for Loading and Boundary Conditions | X |  | See attached details for individual loading and boundary conditions |
|  | Primary output variables | X |  | Position and deformation of the medial meniscus as quantified by volume visualization and deformed surface curvature |
|  | Secondary output variables | X |  | Contact force and forces in meniscal attachments; stress-strain related variables in meniscus: hydrostatic pressure, von Mises stress, total strain energy density |
|  | Source of anatomy | X |  | 1.5 Tesla MR imaging of specimen in sagittal plane |
|  | Modality | X |  | 3D gradient recalled echo (GRE) fast sequence (TE: 3.6 ms, TR: 8.7 ms, 1NEX, flip angle: 35, scanning time 2.23 min) |
|  | Resolution | X |  | 0.66 mm x 0.66 mm in plane; 1.5 mm slice thickness |
|  | Reference configuration | X |  | Obtained from MRI of unloaded specimen; an anatomical coordinate system was established based on literature |
|  | Subcomponents (subsidiary models) for each, fill in form Reporting Parameters for Subcomponents | X |  | See attached details for individual subcomponents |
|  | Interactions for each, fill in form Reporting Parameters for Interactions | X |  | See attached details for individual interactions |

| REPORTING PARAMETERS FOR FINITE ELEMENT ANALYSIS STUDIES IN BIOMECHANICS (cont.) - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 3: Simulation Structure |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Name of simulation software | X |  | Abaqus Standard |
|  | Version of simulation software | X |  | 6.3 |
|  | Solution strategy |  | X | Although, implied to be implicit from the use of Abaqus Standard |
|  | Numerical algorithms |  | X |  |
|  | Convergence criteria |  | X |  |
|  | Post-processing | X |  | Calculation of meniscal volume using Matlab; deformed surface curvature of meniscus with supporting citation for calculations |
| Group 4: Verification |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Methods of verification |  | X |  |
|  | Correctness of formulation |  | X |  |
|  | Comparisons with known solutions |  | X |  |
|  | Sensitivity to simulation settings |  | X |  |
|  | Mesh convergence | X |  | Overall mesh convergence study exploring 7 meshes at various densities with 10% relative change criteria; variables included a subset of primary and secondary outputs |
|  | Assessment of repeatability |  | X |  |
| Group 5: Validation |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Overall modeling assumptions | X |  | Some provided, e.g. rigid body motion of bones |
|  | Physiological relevance of loading and boundary conditions | X |  | While not representative of a daily activity, loads used were similar to those in a clinical setting to examine knee with ACL deficiency |
|  | Justification of parameters | X |  | All utilized material properties are supported by citations, in particular a referral to a factorial design study for the meniscus; selection of meniscal properties also met the requirement of a positive definite stiffness tensor as advised by simulation software manual |

| REPORTING PARAMETERS FOR FINITE ELEMENT ANALYSIS STUDIES IN BIOMECHANICS (cont.) - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 5: Validation (cont.) |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Data used for model development | X |  | Specimen-specific geometry from the MRI of unloaded specimen; tibiofemoral kinematics from MRIs of loaded specimen; literature based material properties |
|  | Data used for model comparison | X |  | Meniscal motion and deformation from MRIs of loaded specimen used for model development |
|  | Validated output | X |  | Primary outputs: position and deformation of meniscus |
|  | Validation procedure | X |  | Direct validation between the experiment results and model predictions of the same specimen |
|  | Sensitivity analysis | X |  | Brief mention of the high sensitivity of meniscal position on modulus of peripheral attachment |
|  | Predictive capacity | X |  | For given tibiofemoral kinematics, prediction of meniscal motion, deformation, and stress distribution |
| Group 6: Availability |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Contact | X |  | Full contact information for corresponding author |
|  | Licensing |  | X |  |
|  | Website for downloads |  | X |  |
|  | Website for development |  | X |  |
|  | Website for public commentary, responses and rating |  | X |  |

| REPORTING PARAMETERS FOR LOADING AND BOUNDARY CONDITIONS - please fill in a new form for each loading and boundary condition - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Loading and Boundary Conditions |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Type | X |  | Rigid body kinematics |
|  | Region | X |  | Tibia centroid |
|  | Magnitude and direction | X |  | Motion of tibia centroid (all six degrees of freedom) relative to femur centroid at different levels of anterior loading were provided |
|  | Time history | X |  | As a function of different anterior loading applied to the specimen |

| REPORTING PARAMETERS FOR LOADING AND BOUNDARY CONDITIONS - please fill in a new form for each loading and boundary condition - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Loading and Boundary Conditions |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Type | X |  | Rigid body kinematics |
|  | Region | X |  | Femur centroid |
|  | Magnitude and direction |  | X | Model description implies prescription of movements of both femur and tibia centroids but only relative tibial motion was reported; femur may be assumed as fixed during simulations |
|  | Time history | X |  | As a function of different anterior loading applied to the specimen |

| REPORTING PARAMETERS FOR SUBCOMPONENTS - please fill in a new form for each subcomponent - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Subcomponents |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Name | X |  | Femur |
|  | Geometric representation | X |  |  |
|  | Source | X |  | Defined from the nodes of the back surface of tibia cartilage; centroid location not reported |
|  | Resolution |  | X | Presumably same as femur cartilage |
|  | Segmentation procedure |  | X | Presumably same as femur cartilage |
|  | Representative geometry |  |  | N/A |
|  | Mesh | X |  |  |
|  | Element name |  | X |  |
|  | Element shape | X |  | 2D rectangular |
|  | Element formulation | X |  | Rigid elements |
|  | Mesh density |  | X | Presumably same as the back face of femur cartilage |
|  | Constitutive representation | X |  | Rigid (as prescribed by element formulation) |
|  | Formulation |  |  | N/A |
|  | Coefficients |  |  | N/A |
|  | Source |  |  | N/A |
|  | Reference configuration |  | X |  |
|  | Other properties |  | X |  |

| REPORTING PARAMETERS FOR SUBCOMPONENTS - please fill in a new form for each subcomponent - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Subcomponents |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Name | X |  | Tibia |
|  | Geometric representation | X |  |  |
|  | Source | X |  | Defined from the nodes of the back surface of tibia cartilage; centroid location not reported |
|  | Resolution |  | X | Presumably same as tibia cartilage |
|  | Segmentation procedure |  | X | Presumably same as tibia cartilage |
|  | Representative geometry |  |  | N/A |
|  | Mesh | X |  |  |
|  | Element name |  | X |  |
|  | Element shape | X |  | 2D rectangular |
|  | Element formulation | X |  | Rigid elements; no other specifics |
|  | Mesh density |  | X | Presumably same as the back face of tibia cartilage |
|  | Constitutive representation | X |  | Rigid (as prescribed by element formulation) |
|  | Formulation |  |  | N/A |
|  | Coefficients |  |  | N/A |
|  | Source |  |  | N/A |
|  | Reference configuration |  | X |  |
|  | Other properties |  | X |  |

| REPORTING PARAMETERS FOR SUBCOMPONENTS - please fill in a new form for each subcomponent - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Subcomponents |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Name | X |  | Femur cartilage |
|  | Geometric representation | X |  |  |
|  | Source | X |  | Same as source of anatomy |
|  | Resolution | X |  | Same as source of anatomy |
|  | Segmentation procedure | X |  | Manual selection of points on individual images; thin-plate smoothing spline fit through scattered points on surfaces, with an error estimate of 0.66 mm |
|  | Representative geometry |  |  | N/A |
|  | Mesh | X |  |  |
|  | Element name |  | X |  |
|  | Element shape | X |  | Pentahedral or hexahedral |
|  | Element formulation |  | X |  |
|  | Mesh density | X |  | Characteristic element size of 1.5 mm in A/P and M/L directions with three layers of element in the depth direction |
|  | Constitutive representation | X |  |  |
|  | Formulation | X |  | Isotropic, linear elastic, homogeneous |
|  | Coefficients | X |  | E=0.5 MPa, ν = 0.35 |
|  | Source | X |  | Experimentation literature |
|  | Reference configuration |  | X |  |
|  | Other properties |  | X |  |

| REPORTING PARAMETERS FOR SUBCOMPONENTS - please fill in a new form for each subcomponent - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Subcomponents |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Name | X |  | Tibia cartilage |
|  | Geometric representation | X |  |  |
|  | Source | X |  | Same as source of anatomy |
|  | Resolution | X |  | Same as source of anatomy |
|  | Segmentation procedure | X |  | Manual selection of points on individual images; thin-plate smoothing spline fit through scattered points on surfaces, with an error estimate of 0.66 mm |
|  | Representative geometry |  |  | N/A |
|  | Mesh | X |  |  |
|  | Element name |  | X |  |
|  | Element shape | X |  | Pentahedral or hexahedral |
|  | Element formulation |  | X |  |
|  | Mesh density | X |  | Characteristic element size of 1.5 mm in A/P and M/L directions with three layers of element in the depth direction |
|  | Constitutive representation | X |  |  |
|  | Formulation | X |  | Isotropic, linear elastic, homogeneous |
|  | Coefficients | X |  | E=0.5 MPa, ν = 0.35 |
|  | Source | X |  | Experimentation literature |
|  | Reference configuration |  | X |  |
|  | Other properties |  | X |  |

| REPORTING PARAMETERS FOR SUBCOMPONENTS - please fill in a new form for each subcomponent - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Subcomponents |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Name | X |  | Meniscus |
|  | Geometric representation | X |  |  |
|  | Source | X |  | Same as source of anatomy |
|  | Resolution | X |  | Same as source of anatomy |
|  | Segmentation procedure | X |  | Manual selection of points on individual images; thin-plate smoothing spline fit through scattered points on surfaces, with an error estimate of 0.66 mm |
|  | Representative geometry |  |  | N/A |
|  | Mesh | X |  |  |
|  | Element name |  | X |  |
|  | Element shape | X |  | Hexahedral |
|  | Element formulation |  | X |  |
|  | Mesh density | X |  | Characteristic element size of 1 mm in all directions with one layer of elements in the inner edge and eight layers of elements in the outer edge |
|  | Constitutive representation | X |  |  |
|  | Formulation | X |  | Transversely isotropic, linear elastic, homogeneous |
|  | Coefficients | X |  | Eθ=125 MPa, ER= EZ =27.5 MPa, νθR= νθZ =0.1, νRZ= 0.33, GθR= GθZ =2 MPa, |
|  | Source | X |  | Experimentation and modeling literature supported by a previously conducted factorial design study |
|  | Reference configuration |  | X |  |
|  | Other properties |  | X |  |

| REPORTING PARAMETERS FOR SUBCOMPONENTS - please fill in a new form for each subcomponent - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Subcomponents |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Name | X |  | Anterior horn attachment |
|  | Geometric representation | X |  |  |
|  | Source | X |  | Same as source of anatomy |
|  | Resolution | X |  | Same as source of anatomy |
|  | Segmentation procedure | X |  | Manual digitization of insertion sites |
|  | Representative geometry | X |  | 59 lines with no specific reporting of insertion sites |
|  | Mesh | X |  |  |
|  | Element name |  | X |  |
|  | Element shape | X |  | Lines |
|  | Element formulation | X |  | Linear spring elements |
|  | Mesh density |  | X |  |
|  | Constitutive representation | X |  |  |
|  | Formulation | X |  | No-compression, linear in tension, incorporating overall modulus (E), cross-sectional-area (A), initial length (L0), and initial strain (ε1) |
|  | Coefficients | X |  | E= 600 MPa, A= 17.8 mm2, L0= 10.5 mm, ε1= -0.05 |
|  | Source | X |  | Geometric variables from MRI (as in source of anatomy); material properties from experimentation literature on human meniscofemoral ligament |
|  | Reference configuration | X |  | Assumed initial strain |
|  | Other properties |  | X |  |

| REPORTING PARAMETERS FOR SUBCOMPONENTS - please fill in a new form for each subcomponent - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Subcomponents |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Name | X |  | Posterior horn attachment |
|  | Geometric representation | X |  |  |
|  | Source | X |  | Same as source of anatomy |
|  | Resolution | X |  | Same as source of anatomy |
|  | Segmentation procedure | X |  | Manual digitization of insertion sites |
|  | Representative geometry | X |  | 59 lines with no specific reporting of insertion sites |
|  | Mesh | X |  |  |
|  | Element name |  | X |  |
|  | Element shape | X |  | Lines |
|  | Element formulation | X |  | Linear spring elements |
|  | Mesh density |  | X |  |
|  | Constitutive representation | X |  |  |
|  | Formulation | X |  | No-compression, linear in tension, incorporating overall modulus (E), cross-sectional-area (A), initial length (L0), and initial strain (ε1) |
|  | Coefficients | X |  | E= 600 MPa, A= 19.1 mm2, L0= 6.7 mm, ε1= -0.05 |
|  | Source | X |  | Geometric variables from MRI (as in source of anatomy); material properties from experimentation literature on human meniscofemoral ligament |
|  | Reference configuration | X |  | Assumed initial strain |
|  | Other properties |  | X |  |

| REPORTING PARAMETERS FOR SUBCOMPONENTS - please fill in a new form for each subcomponent - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Subcomponents |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Name | X |  | Peripheral attachment |
|  | Geometric representation | X |  |  |
|  | Source | X |  | Same as source of anatomy |
|  | Resolution | X |  | Same as source of anatomy |
|  | Segmentation procedure | X |  | Manual digitization of insertion sites |
|  | Representative geometry | X |  | 74 lines distributed over the outer edge of the meniscus without further specifics |
|  | Mesh | X |  |  |
|  | Element name |  | X |  |
|  | Element shape | X |  | Lines |
|  | Element formulation | X |  | Linear spring elements |
|  | Mesh density |  | X |  |
|  | Constitutive representation | X |  |  |
|  | Formulation | X |  | No-compression, linear in tension, incorporating overall modulus (E), cross-sectional-area (A), initial length (L0), and initial strain (ε1) |
|  | Coefficients | X |  | E= 5.5 MPa, A= (0.66 mm thickness) x (77.3 mm length), L0= 5.4 mm, ε1= 0.0 |
|  | Source | X |  | Geometric variables from MRI (as in source of anatomy); material properties from experimentation literature on human hip capsule |
|  | Reference configuration | X |  | Assumed initial strain |
|  | Other properties |  | X |  |

| REPORTING PARAMETERS FOR INTERACTIONS - please fill in a new form for each interaction - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Interactions |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Interacting components | X |  | Femur cartilage – tibia cartilage |
|  | Interaction type | X |  | Contact |
|  | Interaction formulation | X |  | Finite sliding; no specifics relevant to exact formulation for contact resolution other than no penetration allowed |
|  | Interaction properties | X |  | Frictionless; adjustments to prevent initial penetrations, no specific identification of master and slave surfaces |

| REPORTING PARAMETERS FOR INTERACTIONS - please fill in a new form for each interaction - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Interactions |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Interacting components | X |  | Femur cartilage – meniscus |
|  | Interaction type | X |  | Contact |
|  | Interaction formulation | X |  | Finite sliding; no specifics relevant to exact formulation for contact resolution other than no penetration allowed |
|  | Interaction properties | X |  | Frictionless; adjustments to prevent initial penetrations, no specific identification of master and slave surfaces |

| REPORTING PARAMETERS FOR INTERACTIONS - please fill in a new form for each interaction - for description of individual items refer to the manuscript - provide a summary if checked (Yes) for Reported |  |  |  |  |
| --- | --- | --- | --- | --- |
| Group 2: Model Structure - Interactions |  |  |  |  |
|  | ITEM | REPORTED |  | SUMMARY |
|  |  | Yes | No |  |
|  | Interacting components | X |  | Tibia cartilage – meniscus |
|  | Interaction type | X |  | Contact |
|  | Interaction formulation | X |  | Finite sliding; no specifics relevant to exact formulation for contact resolution other than no penetration allowed |
|  | Interaction properties | X |  | Frictionless; adjustments to prevent initial penetrations, no specific identification of master and slave surfaces |