Multi-Scale Tissue Mechanics Modeling Issues

| Scale | Modeling Need | Method/Type | Software | Issues | Possible Shared Items |
| --- | --- | --- | --- | --- | --- |
| Organ Systems | Determination of organ level mechanical loading Musculoskeletal model sensitivity studies Neuromusculoskeletal model optimization studies (Muscle activation patterns) 1 | Multi- body musculoskeletal model (lumped parameter) | MSC.ADAMS SIMM/Dynamics Pipeline/SDFAST | - Difficult to represent soft tissues in non-FE models - Kinematic/kinetic model of joint should be adequate to represent organ level loading , e.g. is it possible to use knee BCs and loading obtained using a hinge description in a follow-up complex finite element model of the knee? | - Simulation Results - Tissue loading, ligament strain, muscle activation patterns - Solution algorithms |
| Organ | Need for computationally efficient models that capture dynamic tissue behavior | Artificial Neural Network Support Vector Machines Surrogate modeling based on adaptive sampling and local interpolation | MATLAB Neural Network Toolbox SPIDER (a free machine learning toolbox for Matlab) | - Space and time complexities - Stability - Generalization | - Modeling Technique - Journal Article - ANN models that capture desired responses from computationally intensive techniques - Modular application programming interface - IDE compatibility - Solution database of complex models |
| Organ | Convert MR/CT images to 3-D geometries | N/A | Slicer Mimics Materialise Volsuite | - Thresholding techniques typically do not work well on musculoskeletal tissues - converted geometries typically need smoothing - integrity of tissue geometries/thickness maintained? | - Tissue Geometries - STL files - Patient Specific - IGS files |
| Tissue | Meshed tissue geometries | N/A | Cubit Patran TrueGrid | - Irregular surfaces difficult to mesh particularly using hexahedral elements - STL format not optimal for meshing - Conversion of STL to NURB surfaces? - Automated generation of tetrahedral meshes is possible but those may not be suitable for nonlinear simulations - Are meshfree methods a solution? | - FE meshed geometries of tissues - Parametric meshing scripts |
| Tissue | Accurate representation of Non-linear, Non-homogenous, Viscoelastic tissues | Finite - Element Inverse finite element method | ABAQUS | - FE method is computationally intensive for dynamic simulations - meshing - contacts - need for accurate constitutive equations therefore complete experimental data sets - material heterogeneity | - Modeling Technique - Journal Article - Constitutive equations - Experimental data to develop constitutive models |
| Multi-cell | Develop constitutive equations of tissue | Pseudo – Bond | Custom | - Material parameter estimation | - Modeling Technique - Journal Articles - Constitutive equations of tissues - Platform specific ? |
|  |  | Boundary Condition coupling |  |  | - Modeling algorithms that couple 1D and 3D lung models |

Guess, Lin, Bischoff, Erdemir,

1 For a review of current methods, see "Erdemir, A., McLean, S., Herzog, W. and van den Bogert, A. J. (2007) Model-based estimation of muscle forces exerted during movements, Clinical Biomechanics, 22, 131-154". I'll be happy to upload the article to SimTk site. I wonder if we violate any publisher copyright rules if we upload original articles though.